|  | CFAS Enterprises Inc. After Market Utility Power Equipment Brokerage Mailto:Staff@CFASPower.com Turbine URL: http://CFASPower.com/ Reciprocating URL: http://CFASPower.com/Recips.html |
| --- | --- |
| GTG_1730TC_Mars100_50Hz.doc |  |

1 x Unused Solar Mars 100 10Mw 50Hz 11KV Duel Fuel Generator Set w/ SoloNox

Type:				SoLoNox

Fuel:				Natural Gas And Light Oil

Electrical Output:		10,6 MW

Voltage:			11 kV, 50 Hz (or 60 Hz for same price)

Condition:			Unused YOM 2007

Available;			20 Days from PO

Emissions:			25 ppm NOx, 50 ppm CO, 25 ppm UHC

Asking Price:		USD $5.25 Million Dollars

Equipment Delivered Ex-Works In Hamburg (In The Tax Free Zone Of A Warehouse).

Scope of supply:

- Complete Package

- Basis Enclosure

- Exhaust Silencer

- Air inlet Ducting

- Silencer

- Inlet Ventilation Silencer

- Exhaust Ventilation Silencer

- Dust Protection

- Barrier Type Filters

- Interior Lighting

- Fire Cylinder Cabinet

- Combustion Gas Monitoring System

Technical Description:

Gas Turbine

• Mars® 100 Industrial, Two-Shaft

• Axial Compressor, 15 Stages

- Variable Geometry Inlet Guide Vanes

- Vertically Split Case

- Compression Ratio: 		17.4:1

- Max. Speed:				11,168 rpm

• Annular Combustion Chamber

- Dry, Low Emission ( SoLoNOx)

- 14 Fuel Injectors ( SoLoNOx)

• Proximity Probe Vibration Transducers

• Gas Producer Turbine

- 2 Stage, Axial Flow

- Max. Speed: 11,168 rpm

• Power Turbine

– 2 Stages, Axial Flow

– Speed: 8625 or 8568 rpm

(50 or 60 Hz)

Main Reduction Drive

• Epicyclical Star Gear

– 1500 or 1800 rpm (50 or 60 Hz)

– Acceleration Vibration Transducer

Generator

• Continuous Duty Rating

• Salient Pole, 3 Phase, 6 Wire,

Wye Connected, Synchronous with

Brushless Exciter

• Open Drip-Proof Construction

• Sleeve Bearings

Solid-State Voltage Regulation with

Permanent Magnet Generator

• Velocity Vibration Transducers

• NEMA Class F Insulation with F Rise

Package

• Steel Base Frame with Drip Pans

• Direct-Drive AC Start System

• Natural Gas and light oil Fuel System

• Control System

– Microprocessor-Based PLC

– Gas Turbine and Generator Control

– Vibration and Temperature Monitoring

- Auto Synchronizing

• Integrated Lube Oil System

– AC Motor-Driven, VFD-Controlled

Lube Pump

– Pre/Post and Backup Lube Pumps

– Air/Oil Cooler

– Integral Lube Oil Tank

– Lube Oil Tank Heater

- Lube Oil Filter

• Documentation

– Drawings

– Quality Control Data Book

– Inspection and Test Plan

– Test Reports

- Operation and Maintenance Manuals

• Factory Testing of Turbine and Package

Optional Equipment/ Services

• NEC Electrical System

• Generator Options:

– Standby Duty Rating

– TEWAC; Weather Protected II

– Standard Voltages:

3300, 6600, 11,000 V (50 Hz);

4160, 6900, 12,470, 13,800 V (60 Hz)

• Fuel Systems

– Liquid or Dual (Gas/Liquid)

– Alternate Fuels (such as naphtha,

propane, low Btu gas)

• Lube Oil System

– Water/Oil Lube Oil Cooler

– Electrostatic Demister

- Duplex Lube Oil Filters

• Control System

– Remote Display/Control Terminal

– Heat Recovery Application Interface

– Serial Link Supervisory Interface

– KW Control

– KVAR/Power Factor Control

– Turbine Performance Map

– Historical Displays

– Printer/Logger

– Predictive Emissions Monitoring

- Field Programming Terminal

• Accessory Equipment

– 120-VDC Battery/Charger System

– Turbine Cleaning System: On-Crank

and On-Line

- Package Lifting Kit

• Weather-proof Acoustic Enclosure

• Various Air Inlet and Exhaust Systems

– Inlet and Exhaust Silencers

– Self-Cleaning or Prefilter/Barrier

Air Inlet Filter